ESCUELA POLITECNICA SUPERIOR ´

INTRODUCTION TO FLUID MECHANICS. 20-01-2015 PROBLEM 1 (10 POINTS). Time: 60 minutes

NAME:...................................................................................... GROUP:.......

The cylindrical tank in the figure, of diameter D and height H, is supplied with a constant volume flux Q of a liquid of density ρ through a circular pipe of diameter d � D placed at the bottom of the tank. A valve in the feeding pipe is closed when the float A attains a horizontal position. The float, of volume V and density ρv > ρ/2, is connected to a joint at point O through a massless rod of length l. The size of the float is small compared to all relevant dimensions in the problem.

Using the control volume Vc plotted in the figure with dashed lines,

1. Determine the time necessary to completely fill the tank if the float A is initially at position Z = 0.

2. Obtain the vertical force Fz and the torque M about the point O exerted by the float A on the rod. To do that, apply Newton’s second law to the float A knowing that half of its volume (that is V /2) is submerged in the liquid. Bear in mind that the free surface velocity is dh/dt and consider the fluid at rest relative to the float.

3. Compute the pressure at the bottom of the tank p0 = p(z = 0), as a function of the height h. The fluid velocity is vertical all over the tank, except for a small region (remember d � D) near the entrance.

4. Obtain the horizontal force that the fluid exerts on the cylindrical wall of the tank knowing that the pressure of the fluid that enters the tank through the feeding pipe is equal to the pressure at the bottom of the tank.

NOTE: The ambient air pressure is pa.

O

�

Fz z l

g

h

A

H

Q

x VcD

ESCUELA POLITECNICA SUPERIOR ´

INTRODUCTION TO FLUID MECHANICS. 20-01-2015 PROBLEM 2 (10 POINTS). Time: 60 minutes

NAME:...................................................................................... GROUP:.......

Consider an incompressible fluid with constant density ρ, specific heat c and thermal conductivity k. The viscosity µ(T) is a function of tthe temperature given by

µ(T) = µ0

� T T0

�3/2 ,

where µ0 and T0 are constants. This fluid is moving between two infinitely long horizontal walls a distance h apart. Both walls are at rest. The fluid flow is two-dimensional, in the plane (x, y), where y is the coordinate perpendicular to the walls. The temperature of the lower wall is T0 and the temperature of the upper wall is 2T0. If the velocities and temperature are function only of the distance to the walls:

1. Find the vertical velocity component v(y)

2. Obtain the temperature field T(y). Neglect viscous dissipation.

3. Write the viscous stress tensor τ � as a function of the distance to the walls, u(y) and its deriva tives.

4. Prove that ∂P

∂xis constant.

5. Obtain the velocity component u(y), assuming ∂P

∂xis known.

NOTE:� ξ + C

(1 + ξ)3/2 = 4 + 2ξ − 2C

√1 + ξ

ESCUELA POLITECNICA SUPERIOR ´

INTRODUCTION TO FLUID MECHANICS. 20-01-2015 PROBLEM 3 (10 POINTS). Time: 60 minutes

NAME:...................................................................................... GROUP:.......

Consider a reactive gas stored in a spherical container of radius a and placed in a furnace kept at temperature T0. The spontaneous exothermic reaction of the gas releases an amount of heat per unit time and per unit volume Q.

Under weakly reactive conditions, the temperature increment ΔT induced in the gas has the following parametric dependence:

ΔT = f(a, k, T0, Q),

where k is the gas thermal conductivity.

Experimental results are obtained from the thermal decomposition of azomethane (thermal con ductivity k = 0.05 W/(m K)) in a spherical reactor of radius a = 6 cm kept at an ambient temperature of T0 = 650 K. By changing the initial concentration of the gas, different values of Q are obtained and, for each value of Q, the temperature increment is measured at the center of the reactor. The data is given below together with its graphic representation:

| a = 6 cm T0 = 650 K k = 0.05 W/(m K) |  |
| --- | --- |
| Q (W/m3) | ΔT (K) |
| 295 502 652 757 828 871 893 900 | 4.1 8.0 11.9 15.8 19.7 23.6 27.5 31.4 |

ΔT (K)

40 30 20 10

0 200 400 600 800 1000 0

Q (W/m3)

In the experiments, larger values of Q produce a thermal explosion and no stable quasi-steady conditions are attained. The conditions of the last point in the curve determine the critical point.

1. Apply dimensional analysis techniques and write the functional relation in dimensionless terms.

2. From the available experimental results, obtain a curve applicable to any reactive system in a spherical container of any size.

3. If a reactor of 10 cm of radius is kept at an ambient temperature of 700 K with azomethane in conditions to release 337.7 W/m3, what would be the temperature increment measured at the center?

4. For azomethane that releases Q = 300 W/m3 in an ambient temperature of T0 = 700 K, which would be the critical radius of the reactor to produce thermal explosion? and what would be the temperature increment in that case?

ESCUELA POLITECNICA SUPERIOR ´

INTRODUCTION TO FLUID MECHANICS. 20-01-2015 PROBLEM 4 (10 POINTS). Time: 60 minutes

NAME:...................................................................................... GROUP:.......

We want to push the cylindrical body shown in the figure against the wall with a constant vertical velocity vB. A cylindrical volume of oil, of initial radius R/2 and height h0 equal to the initial separation of the body and the wall, exists between the body and the wall. As the body approaches the wall, the volume of oil will deform, increasing its radius l but maintaining the cylindrical shape. when h decreases below h0/4, the radius of the oil volume is kept constant l = R by bleeding the necessary volumetric rate Q through a tiny hole in the wall, as sketched in the figure. If the density and viscosity of the oil are, respectively, ρ and µ:

1. Give the condition to be satisfied by the vertical velocity vB for the flow to be dominated by viscosity.

2. Find the radial velocity profile as a function of the radial pressure gradient ∂P ∂r

For h0 ≥ h ≥ h0/4:

3. Derive an equation for the pressure distribution, with the necessary boundary conditions. Find P(r, t).

4. Obtain the force exerted by the fluid on the body, specifying clearly the direction. For h0/4 > h:

5. Find the volumetric rate Q that must be extracted to maintain the radius of the oil volume equal to R.

6. Derive an equation for the pressure distribution, with the necessary boundary conditions. Find P(r, t). Notice that special treatment needs to be done for r → 0, and the pressure will be singular there.

7. Find the force exerted by the fluid on the body in this case. Notice that the singularity already mentioned is integrable.

Suggestion: Use the manometric reduced pressure P.

vB

ρ µ

l

h ≥ h0/4

R

h

R

vB

ρ µ

Q

h < h0/4